Solar Sun World PowerHub

Renewable

Backup Power System

Watts

1800

A Clean, Green, Backup Power Solution

The PowerHub is a combination of an inverter/charger module capable of delivering

up to 1800 watts of household power.

It is the first backup power solution to operate

with solar and wind inputs.

Power from a generator or the utility grid can also be used with

the PowerHub system to recharge its batteries. The PowerHub provides an affordable, high-

output, primary power solution for an off-grid cottage or cabin, as well as backup power for

grid-connected homes when the power goes out. It is scalable up to 400 amp-hours of energy,

allowing customers to build their system based on their power requirements.

Never worry about running out of power – renewable power sources, solar and wind, sustain

the PowerHub’s battery charge indefinitely

An excellent alternative to a generator – designed for indoor use, with no fuel, fumes or noise

Add it to a grid-tie solar energy system for backup power

Product Features

Modified sine wave inverter/charger with battery box

The battery box stores up to 200 amp-hours of battery power

(

single battery box included; batteries purchased separately

)

A second battery may be connected to double the battery capacity

(

additional battery box and batteries sold separately

)

Designed to include a generator as part of the system; generator power can pass

through the unit to the AC outlets, and recharge the batteries

Integrated transfer switch

40-

amp three-stage charger

Four AC outlets with hardwire option

Two DC inputs for solar and/or wind connections

Full-feature LED display for system monitoring

Solar Sun World

2701 Little River Road

Madison, Ga 30650

Office: 706-752-1682

Cell: 706-318-0784

jk@solarsunworld.com

www.solarsunworld.com

| Battery Charger Specifications |  |
| --- | --- |
| Output current | 0/10/40 A |
| Output voltage |  |
| Charge | 14.2 Vdc |
| Float | 13.7 Vdc |
| Recharge | 12.5 Vdc |
| AC input voltage | 115 Vac 50/60 Hz |
| Charger efficiency | 80% approximate |
| Charging type | Three-stage |

| PowerHub 1800 Runtimes (single battery box with 200 AH battery) |  |  |
| --- | --- | --- |
| Office Applications | Watts | Runtime |
| Laptop | 65 | 22 h |
| Inkjet printer | 8 | 218 h 20 min |
| Cell phone | 5 | 396 h |
| Table lamp | 25 | 54 h 45 min |
| 17” LCD monitor | 35 | 49 h 50 min |
| Table light 40 W | 40 | 43 h |
| Table light 60 W | 60 | 26 h 40 min |
| Emergency Power Applications | Watts | Runtime |
| Cordless phone | 5 | 396 h |
| Clock radio | 8 | 218 h 20 min |
| 13” CRT TV | 50 | 30 h 10 min |
| 20” LCD TV | 370 | 3 h 20 min |
| Home Backup Power Applications | Watts | Runtime |
| 18 cu. ft. fridge | 120 | 15 h 25 min |
| 8.8 cu. ft. freezer | 80 | 20 h 20 min |
| Sump pump (1/2 hp) | 300 | 4 h 29 min |
| Home alarm system | 5 | 396 h |
| Microwave | 1000 | 1h 43 min |
| Coffee maker | 1200 | 1h |

www.solarsunworld.com

706-752-1682

• jk@solarsunworld.com

€